- NEWA8051.DOC -

A8051 2.00 Release Notes

========================

Created: 10/Jul/90 [IANR]

Dear User;

This file contains important information which is not available in the current

manual.  PLEASE READ IT (as this may save you a lot of time)!

This release of the A8051 cross assembler is upwardly compatible with previous

versions.  The enhancements are entirely devoted to support the 4.00 version

of IAR's 8051 C compiler.  The added directives are therefore only specified

in the 8051 C manual.

Other changes:  The MS-DOS version now has a symbol-table capability in excess

of 1000 symbols on a machine with 512K user RAM.

Linker change:  In XLINK revisions above 4.20 the linker algorithm has been

changed for segments prefixed with (BIT), (DATA) or (IDATA) which requires

changes in linker command files for programs using reloctable bit and byte

addressable elements.

THIS MEANS THAT SECTION 1.11 OF THE ASSEMBLER APPENDIX IS NO LONGER VALID!

With XLINK < 4.20:   (as described in section 1.11)

-Z(IDATA)BYTESEG=20

-Z(BIT)BITSEG=0

-z

With XLINK >= 4.20

-ZBYTESEG=20

-Z(BIT)BITSEG=0

-z

If there are other bit or byte segments as with ICC8051 the following

should be performed:

-Z(BIT)BITSEG,OTHER_BIT_SEGS=0

-ZBYTESEG=20

-z

-Z(DATA)DIRECT_SEGS=8

-Z(IDATA)INDIRECT_SEGS

I.e. there should only be one list of (BIT) segments and BITSEG should be

first (to make sure it is located to zero).

-ZBYTESEG should be alone and set to the start address of the bit-addressable

user area of the 8051.

If there are DATA (directly addressable memory) segments they should be in a

separate list with a valid start address.

If there are IDATA (indirectly addressable memory) segments they should be in

a separate list with a valid start address (unless there were DATA segments

whos end address gives the start of IDATA).