The IFP achitectures

The main line of development of von Neumann processors

with reference to parallel instruction processing

Overview of IFP processors

###	At pipelined operation instruction processing is subdivided into a number of successive subtasks, such as instruction fetch (F), decode, (D), execute (E) and writeback of the result (W)

###	At parallel operation a number of functional units (FU), each of them capable of parallel operation on data fetched from a set of registers. Now, multiple FUs execute instructions and write back results into the register file in parallel.

Pipeline stages

a: Subtasks of instruction execution for integer register-register operations

b: Basic structure of an integer pipeline

c: Timing of the execution

All the pipeline stages operate like an assembly line, with synchron timing of the operation. At the beginning of every pipeline cycle each stage accepts a new input and at the same time it delivers its output to the next stage, except the last one, which writes the result into the specified destination

Parallel execution of instructions

in a traditional pipelined processor

|  | t1 | t2 | t3 | t4 | t5 | t6 | t7 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | t |  |
| instr. 1 | F1 | D1 | E1 | W1 |  |  |  |  |  |
| instr. 2 |  | F2 | D2 | E2 | W2 |  |  |  |  |
| instr. 3 |  |  | F3 | D3 | E3 | W3 |  |  |  |
| instr. 4 |  |  |  | F4 | D4 | E4 | W4 |  |  |
| instr. 5 |  |  |  |  | F5 | D5 | E5 |  |  |
| instr. 6 |  |  |  |  |  | D6 | D6 |  |  |
| instr. 7 |  |  |  |  |  |  | F7 |  |  |

It is a significant feature of pipelined execution that in each cycle a new instruction can enter the pipeline. Evidently, if an instruction has completed the last phase of its execution, it retires. Thus, as many instructions can be executed in parallel, as there are pipeline stages, that is four in our example.

Number and kind of pipeline stages

in the pipelined execution units of the PowerPC 601

| Branch instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fetch | Dispatch Decode Execute Predict |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Integer instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fetch | Dispatch Decode |  | Execute |  |  |  | Writeback |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Load/store instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fetch | Dispatch Decode |  | Addr gen |  |  |  | Cache |  |  |  |  | Writeback |  |  |  |  |  |  |  |  |  |  |
| Floating-point instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fetch | Dispatch |  | Decode |  |  |  | Execute 1 |  |  |  |  | Execute 2 |  |  |  |  | Writeback |  |  |  |  |  |

Pipelining offers higher performance at a cost of increased hardware complexity. Accordingly, pipelined processors were first employed in very expensive super- and vector processors, around 1970. Subsequently, they became the standard implementation of mainframes and, in the 80s, of microprocessors. At present, pipelined execution units (such as fixed- or floating point execution units) are commonly used as building blocks of advanced processors, like VLIW or superscalar ones. For example, the PowerPC 601 has three pipelined execution units, each one for fixed-point, floating-point- and branch execution. The number of required pipeline stages depends, first of all, on the kind of the operation to be performed. Usually, two to ten stages are used. For instance, the pipelined execution units of the PowerPC 601 use two to six stages.

Common basic structure

of conventional VLIW and superscalar processors

A common feature of VLIW and superscalar processors is that both are made up of a number of execution units (EU), each of them capable of parallel operation on data fetched from a set of registers. Now, multiple EUs execute instructions and write back results into the register file in parallel.

Distinction between VLIW and superscalar processors

The VLIW approach  obviously needs very long instruction words in order to specify what each EU should do. The necessary length of a VLIW instruction is n-times the length of a traditional RISC instruction word length, if n EUs are presumed. Thus, for VLIW processors usually embodying of the order of ten execution units, the required word lengths are of the order of hundreds of bits. For instance, the TRACE VLIW family can execute 7 to 28 instructions in parallel, depending on the number of modules incorporated (1 - 4), and has word lengths of 256 - 1024 bits (Colwell 1987).

On the other hand, superscalar processors receive a sequential stream of conventional instructions. It is then the task of the Decode and issue unit to issue, in each cycle, multiple instructions for the multiple EUs. Current superscalar processors issue usually two (for instance DEC Alpha 21064, Intel Pentium), three (e.g. i960, PowerPC 6O1) four (as the IBM RISC/6000 or PowerPC 604) or seldom more (e.g. 6 in case of the Power2) instructions.

Constraints related to IFP execution

The sequential consistency of the execution is needed to guarantee the same results by parallel execution as would be produced during a traditional sequential run.

-	The resource constraints, termed resource dependences, might also impose restrictions on instruction issue and processing. As an example, consider a processor having two fixed-point units and a single floating-point unit, all pipelined and capable of accepting instructions in each machine cycle. Then, evidently, not more than two fixed-point instructions and a single floating-point instruction can be issued in each cycle. Then, evidently, not more than two fixed-point instructions and a single floating-point instruction can be issued in each cycle.

Kinds of dependences

Data dependences are precedence requirements between instructions, needed to keep data, which are referenced by the instructions, consistent with those of a sequential execution. For instance, a precedence requirement is that a data value has to be produced sooner than it can be used.

Conditional instructions, such as conditional branches, also impose restrictions upon the sequence of instructions, which are referred to as control dependences. For example, in case of a conditional branch, the choice of which succeeding path will be taken, depends obviously, on the result of the evaluation of the referred condition.

The resource dependences, arises from the limited availability of hardware resources.

Basic approaches to instruction scheduling

During the static scheduling the compiler reorganises a code in order to avoid dependences, but beyond that, it also performs a code optimisation to expose as much parallelism to the architecture as possible.

In contrast, dynamic scheduling is performed by the architecture. For this reason, the processor maintains, typically, two gliding windows: the issue and execution window.

The main aspects concerning data dependences

RAW-dependence (Read after Write dependence or flow dependence)

WAR-dependence (Write After Read dependence, anti-dependence)

WAW-dependence (Write After Write dependence, output dependence)

Terms relating to data dependences occurring in

"loop-free code"

In general, an instruction il  is said to be RAW-dependent on another instruction ik, if il references as a source, a data value which will be produced by ik. RAW-dependences are also called flow dependences. They are, at the same time, true dependences, since they cannot be abandoned. This is in sharp contrast to WAR- and WAW-dependences which can be circumvented by proper methods.

Data dependence graphs

Let us now consider a straight line code sequence, such as shown in Figure 5.2.7. Evidently, all the dependences among the instructions can be represented by a directed graph, as indicated in the same figure.

Instruction interpretation:  etc.

Figure 5.2.7: A straight line assembly code sequence and its corresponding DDG

The graph has a node for each instruction considered. Directed arcs indicate dependences (precedence relations) between instruction pairs. In additional, as a clarification, we labelled each arc ###t, ###a or ###o for denoting true (RAW), anti (WAR) or output (WAW) dependence, respectively.

Control dependences

Conditional branches are also a major cause to impede the parallel execution of instructions. For instance, in the code sequence

sub r1, r2, r3;

jz    zproc;

mul r4, r1, r1;

:

:

zproc: load r1, x:

:

the actual path of execution depends on the outcome of the subtraction. This means that the instructions following a conditional branch, are dependent on it. In a similar way, all conditional control instructions, such as conditional branches, calls, skips etc., cause dependences on the subsequent instructions which are referred to as control dependences.

How frequent are conditional branches?

This is a point which deserves special attention in order to see how seriously conditional branches impede performance.

|  |  |  | Branch ratio ### |  |  | Uncond. total branch ratio ### |  |  |  | Uncond. branch ratio ### |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | gen. purp. |  | scientific | gen. purp. |  | scientific |  | gen. purp. |  | scientific |  |
| Yeh, Pat (1992) M88100 |  |  | 24 |  | 5 | 78 |  | 82 |  | 20 |  | 4 |  |
| Stephens & al. (1991) RS/6000 |  |  | 22 |  | 11 | 83 |  | 85 |  | 18 |  | 9 |  |
| Lee, Smith (1984) IBM/370 |  |  | 29 |  | 10 | 73 |  | 73 |  | 21 |  | 8 |  |
|  | PDP-11-70 | 39 |  |  |  |  | 46 |  |  |  | 18 |  |  |
|  | CDC 6400 |  |  | 8 |  |  |  |  | 53 |  |  |  | 4 |

Published branch statistics

The compiled data indicate that general purpose code have a very high percentage of branches, which can amount to 20 - 30 %. By contrast, in scientific floating point programs we can expect much fewer branches, the probable frequency is as low as 5 - 10 %. The ratio of unconditional branches and the total number of branches seems to be quite stable in different programs, remaining within the range of 75 - 85 %. As a consequence, the expected frequency of unconditional branches in general purpose code is about 20 %, whereas in scientific programs it is merely 5 -10 %. This means that as far as IFP processors are concerned, control dependences constitute a more serious hurdle in speeding up a general purpose code than a scientific one. A further consequence is that in considering conditional branches, it makes more sense to distinguish between general purpose and scientific code than discussing average values.

Impact of increasing the instruction issue rate

on the frequency of conditional

Frequent conditional branches, however, impose a heavy performance constraint on IFP architectures. This is due to the fact that IFP processors can boost performance mainly by executing more and more instructions in parallel. Increased parallel execution, on the other hand, forces multiple instruction issue, in order to feed multiple execution units properly. However, the more instructions will be issued, on the average, in each cycle, the higher the probability is to encounter a conditional branch, that is a control dependence, during an instruction issue.

Control dependence graph

Like data dependences, control dependences can also be represented by directed graphs.Although, there are several slightly differing interpretations and designations, basically, in order to represent control dependences, nodes with two successors are introduced. The outgoing arcs now represent the true (T) and the false (F) path, and are usually labelled accordingly, with T and F.

Nodes with only one outgoing arc, are interpreted either as operational instructions or as a sequence of conditional branch free operating instructions (straight-line code). The usual designation of directed graphs expressing control dependences is Control Dependence Graph (CDG).

Figure 5.2.11: An example for a CDG

Resource dependences

An instruction is resource dependent on any previously issued instruction if it requires a hardware resource, in most cases an execution unit, which is in use by a previously issued instruction. If, for instance, there is only a single multiplication unit available, then in the code sequence

mul r1, r2, r3:

mul r4, r2, r5;

the second multiplication instruction is resource dependent on the first and can not be executed in parallel with the first one. By providing a large enough number of high performance execution units resource dependences usually do not reduce seriously the performance of up-to-date IFP architectures.

| Processor type | Year of appearance | Number of execution units |
| --- | --- | --- |
| Motorola 88110 | 1991 | 8 |
| DEC Alpha 21064 | 1992 | 2 |
| Intel Pentium | 1993 | 3 |
| PowerPC 601 | 1993 | 3 |
| PowerPC 604 | 1994 | 5 |

Number of execution units provided by recent superscalar processors

Exception handling of a sequential processor

If a particular instruction causes an exception during its execution, and as a consequence, some time later the execution will be interrupted in order to process the exception, the state of an IFP processor by its interruption, must be the same as a sequential processor would have. Practically, this requirement can be satisfied by an IFP processor by delaying the acceptance of an exception until all instructions preceding the faulty one have been executed, and by preventing all subsequent ones from execution.

How much speed-up potential has functional parallelism ?

The next table summarises a number of the available results concerning the potential speed-up of programs when both, true data and control dependences restrict parallel instruction execution. The published data are divided into two groups: general purpose and scientific programs. Compilers, operating systems and non numeric application programs, are regarded as general purpose programs whereas scientific programs are dedicated ones containing massive floating point computations usually on vectors, or matrices.

| Paper | Benchmark | Speed-up for general programs |  | Speed-up for scientific programs |  |
| --- | --- | --- | --- | --- | --- |
|  |  | range | average | range | average |
| Tjaden, Flynn 1970 | 31 Library programs |  |  | 1.2 - 3.2 | 1.9 |
| Kuck & at. 1972 | 20 FORTRAN programs |  |  | 1.2 - 17 | ### 4 |
| Riseman, Foster 1972 | 7 FORTRAN/ assembly programs | 1.2 - 3.0 | 1.8 | 1.4 - 1.6 | 1.6 |
| Jouppy 1989 | 8 Modula-2 programs | 1.6 - 2.2 | 1.9 | 2.4 - 3.3 | 2.8 |
| Lam, Wilson 1992 | 6 SPECmarks + 4 others | 1.5 - 2.8 | 2.1 | 2 - 293 |  |

The figures show that, subject to the assumptions made, general purpose programs which embody predominantly irregular parallelism, have only a very low amount of available parallelism. Most investigations have revealed an astonishingly low potential speed-up in the range of 1.2 - 3.0 with an average value of about 2. The corresponding speed-up values for scientific programs are a little higher being in the range of 1.2 -17 with an average of about 2 - 4.

Spead-up: handling of control depenences

| Paper | Benchmark | Speed-up for general purpose programs |  |  | Speed-up for scientific programs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | range | average |  | range | average |
| Riseman, Foster 1972 | 7 FORTRAN/ assembly programs |  | 8 - 100 | 42 |  | 30 - 120 | 75 |
| Nicolaus, Fisher 1984 | 22 numerical programs |  |  |  |  | 3 - 988 | 12 |
| Kumar 1988 | 4 FORTRAN programs |  |  |  |  | 475 - 3500 | 1839 |
| Butler & at.1991 | 9 SPEC benchmarks |  | 38 - 200 | 170 |  | 17 - 1165 | 509 |
| Wall 1991 | 6SPECbenchmarks,13 others |  | 16 -41 7 - 37 | 28 24 |  | 57 - 60 6 - 27 | 59 18 |
| Austin, Sohi 1992 | SPEC benchmarks |  | 13 - 942 | 288 |  | 51 - 33749 | 6188 |
| Lam, Wilson 1992 | 6SPEC benchmarks, 4 others |  | 174 - 3283 47 - 265 | 1400 229 |  | 844 - 188 470 | 64500 |

These data indicate that control dependences are the real obstacle in utilising instruction level parallelism. If we were able to make perfect oracles, and could assume the availability of sufficient resources of hardware, we could expect speed-up values of the order of 10 to 100, in some cases even higher. In practice, of course, no oracles can be made, and these are no unlimited resources available.

Spead-up: effect of register renaming I.

The achievable speed-up figures assuming either no elimination, imperfect elimination using 32, 64 or 256 renaming registers, or a perfect elimination assuming an unlimited number of available registers. The results show that false dependence elimination has, for the majority of benchmarks, a considerable impact on increasing speed-up. Therefore, an advanced IFP execution is expected to remove false dependences in one or another form. However, in order to achieve higher values of gain, at least 64 renaming registers appear to be required.

Spead-up: effect of register renaming II.

The characteristics of the test programs used in transparens I.

|  | Lines | Dynamic instructions | Remarks |
| --- | --- | --- | --- |
| Livermore | 268 | 22294030 | Livermore loops 1-14 |
| Whetstones | 462 | 24479634 | Floating-point |
| Linpack | 814 | 174883597 | Linear algebra (3) |
| Stanford | 1019 | 20759516 | Hennessy's suite (5) |
| sed | 1751 | 1447717 | Stream editor |
| egrep | 844 | 13910586 | File search |
| yacc | 1856 | 30948883 | Compiler-compiler |
| metronome | 4287 | 70235508 | Timing verifier |
| grr | 5883 | 142980475 | PCB router |
| eco | 2721 | 26702439 | Recursive tree comparison |
| ccom | 10142 | 18456797 | C compiler front end |
| gcc1 | 83000 | 22745232 | pass 1 of GNU C compiler |
| espresso | 12000 | 135317102 | boolean function minimizer |
| li | 7000 | 1247190509 | Lisp interpreter |
| fpppp | 2600 | 244124171 | quantum chemistry |
| doduc | 5200 | 284697827 | hidrocode simulation |
| tomcatv | 180 | 1986257545 | mesh generation |

Parallelism extraction

The dynamic scheduling comprises only a dependence check as well as the issue of independent instructions. On the other hand, static scheduling includes both dependence check as well as code optimisation. Therefore, we can expect static scheduling to be more effective than dynamic scheduling, as far as parallelism extraction is concerned.

However, static scheduling and, consequently, static parallelism extraction may be performed at three different levels: at basic block-, loop- and global level. The associated scheduling and the embodied parallelism extraction techniques are termed accordingly, as basic block- (or local), loop- and global techniques. These techniques have an increasing performance in the listed order.

An example of basic block and its length

An assembly level basic block and its graphical representation by a DDG

|  | Program | Basic Block Length |  |
| --- | --- | --- | --- |
|  |  | General purpose programs | Scientific programs |
|  | awk | 7.8 |  |
| others | ccom | 7.5 |  |
|  | irsim | 7.7 |  |
|  | latex | 10.4 |  |
|  | eqntott | 4.4 |  |
|  | espresso | 7.0 |  |
| specmarks | qcc (cc1) | 8.9 |  |
|  | matrix 3000 |  | 21.0 |
|  | spice 2g6 |  | 14.1 |
|  | tomcatv |  | 59.8 |
|  | average | 7.8 | 31.6 |

Basic block lengths of 6 SPEC benchmark and 4 other programs, measured on a MIPS 3000

The loop level and the global level extraction

The next possible level of parallelism extraction is loop level extraction. Here, instructions belonging to consecutive iterations of a loop can usually be executed in parallel resulting in considerable speed-up. However, recurrences may, to a certain extent, impede achievable speed-up.

It is widely accepted that loops are a significant source of parallelism, first of all in scientific programs. Therefore, it is apparent, that highly parallel ILP architectures, such as VLIW processors, should extract parallelism  at least at the loop level. Accordingly, a large number of techniques have been developed to perform scheduling at this level.

The most effective way to extract parallelism is to do it at the highest possible level, using global scheduling techniques. Here, parallelism is searched for and extracted beyond basic blocks and simple loops, in such constructs as compound program sections involving loops as well as conditional control constructs. Evidently, in case of a chunk of code larger than a basic block, there are much more possibilities for parallelisation. However, the price to pay is increased complexity.