# SHARED MEMORY MIMD ARCHITECTURES

Etelka Kovács and Károly Kondorosi

1994-1997

# Introduction

·	MIMD architecture is the most popular and general one to build multiprocessor or multicomputer systems consisting of calassical von Neumann-type processors.

·	Other type of processors (dataflow, reduction etc.) can also be used to build MIMD architectures, but these are not discussed in this part.

·	Architectures are discussed mainly at the hardware-software interface layer often called the software modell of the computer. (I.e. the modell, what the instruction set of the computer defines.)

Introduction - 2

Two basic classes of MIMD computers:

·	A Shared Memory System has a Global Memory Address Space accessible for each processor according to the RAM (PRAM) model.

·	In a Distributed Memory System each processor has its own private memory address space not accessible for any other processor, and a message passing mechanism for communication.

# Basic Problems with SM Systems

Structure:

Memory blocks (M0 - Mn):

	huge number of cells

	limited number of access-points

	a single global address space

Processors (P0 - Pk):

	von Neumann type

Interconnection Network:

	crucial point for performance

Problems:

Memory Access Time 	can be a bottleneck even in a single-processor system

Contention for Memory		two or more processors want to access a location in the same block at the same time (hot spot problem).

Contention for Communication		processors should share and use exclusively elements of the Interconnection Network

Result: long latency-time, idle processors, nonscalable system.

How to increase scalability

1. To do something with memory organization

Distributed memory seems to be more efficient; while processors are using their private memory (as it is the case in executing a process with good locality), they will not disturb each other.

Problem: it is mostly left to the users to configure the system efficiently.

Let's apply cache and automatic data-migration based on the old, good principle of locality.

2. To apply efficient Interconnection Network

	Fast (bandwidth)

	Flexible (no unnecessary restriction of multiple concurrent communication)

	Safe (no interference)

	Support for broadcasting and multicasting

3. To do something with idle processors waiting for memory or communication

Using the old, good principle of multiprogramming in a lower-level layer: support for thread-level parallelism within a processor.

# Memory Organization

Ideas:

1. Cache

Provide each processor with a cache memory, and apply an appropriate automatic data-exchange mechanism between the caches and the main memory.

Cache coherence problem.

2. Virtual (or Distributed) Shared Memory

Distribute the global memory to processors. Provide each processor with a private memory, but allow them to access the memory of other processors - as part of a global address space - too.

Physically, these systems have distributed memory architecture, but a logical layer (implemented in HW) simulates a shared memory architecture over them.

NUMA, COMA, CC-NUMA machines

Basic Types of Memory Organization in SM Computers

Uniform Memory Access (UMA):

real (physically) shared memory, no difference in accessing any block in the global address space.

Non-Uniform Memory Access (NUMA):

PE: Processing Element

	Logically shared memory is physically distributed

	Different access of local and remote memory blocks

	Remote access takes much more time - latency

	Sensitive to data and program distribution

Cache-Only Memory Access (COMA):

Each block of the shared memory works as local cache of a processor

Hit-rate decreases the traffic on the Interconnection Network

Solutions for data-consistency increase the same traffic (see cache coherency problem later)

Cache-Coherent Non-Uniform Memory Access (CC-NUMA):

A combination of NUMA and COMA

Cache coherency problem is to be solved

COMA and CC-NUMA are used in newer generation of parallel computers

Using Caches

Effects of cache memory:

Reduced latency (shorter average memory access time)

Reduced traffic on IN

Less chance to wait for communication or memory

Problem of Cache Coherence:

There may exist several copies of the same data in the system, coherence of which should be kept. (Example: P0 has modified x to x', P1...Pk should see x' in all operations executed after the modification.)

Typical Cache Organization